Module Name: top_lvl Calculator

Design Engineers: Pranav Eada + SJ DD Team

Verification Engineers: Pranav Eada + SJ DV Team

Test Bench File: calc_tb_top.sv

| Functionality | Basic Computation |
| --- | --- |
| - Normal addition with no overflow | Inputs: a1=0x12345678, b1=0x87654321 loaded into SRAM_A[5], SRAM_B[5]. Action: calc_driver_h.start_calc(5,6,15,15). Expected: SRAM_A[15]=0x99999999, SRAM_B[15]=[a2+b2]. PASS if matches. |
| - Normal Addition with overflow | Inputs: a1=0xFFFFFFFF, b1=0x00000001 at SRAM_A[10], SRAM_B[10]. Action: calc_driver_h.start_calc(10,11,20,20). Expected: SRAM_A[20]=0x00000000 (wrap-around), SRAM_B[20]=a2+b2. PASS if matches. |

| Functionality | Edge Cases |
| --- | --- |
| - Zero Inputs case | Inputs: SRAM_A[100]=0x0, SRAM_B[100]=0x0; SRAM_A[101]=0x0, SRAM_B[101]=0x0. Action: calc_driver_h.start_calc(100,101,200,200). Expected: SRAM_A[200]=0x0, SRAM_B[200]=0x0. PASS if matches. |
| - Multiple operations | Inputs: SRAM_A[50]=0x11111111, SRAM_B[50]=0x22222222; SRAM_A[51]=0x33333333, SRAM_B[51]=0x44444444; SRAM_A[52]=0x55555555, SRAM_B[52]=0x66666666; SRAM_A[53]=0x77777777, SRAM_B[53]=0x88888888. Action: calc_driver_h.start_calc(50,53,100,101). Expected: Output addresses 100/101 contain correct sums. |

| Functionality | Data Flow and SRAM Integrity |
| --- | --- |
| - Data Read | Verified via monitor/scoreboard. DUT must read from SRAM_A/B within valid address range. |
| - Data Write | Verified after each calculation. Driver + monitor check SRAM_A/B contents against expected sums. |

| Functionality | Timing & Reset Tests (Assertion) |
| --- | --- |
| - Reset functionality | property reset_check; @(posedge clk) rst ¦=> (state == S_IDLE); endproperty RESET: assert property(reset_check); |
| - Verify the buffer_loc toggling logic. | property buffer_toggle; @(posedge clk) disable iff (rst ¦¦ reset_test_active) (state == S_ADD && my_calc.u_ctrl.bufsel_q == 0) ¦=> ##1 (my_calc.u_ctrl.bufsel_q == 1); endproperty BUFFER_LOC_TOGGLES: assert property(buffer_toggle); |
| - Verify the valid input addr | property addr_check; @(posedge clk) my_calc.read ¦-> (my_calc.r_addr >= my_calc.u_ctrl.read_start_addr) && (my_calc.r_addr <= my_calc.u_ctrl.read_end_addr); endproperty VALID_INPUT_ADDRESS: assert property(addr_check); |

| Functionality | Random Constrained Test (Coverage) |
| --- | --- |
| - give random input | Verify the output. Overall Coverage for DUT >= 96% 10 randomized seq_item_t generated with varying read/write ranges. Driver pushes items, DUT executes, monitor/scoreboard compares. Expected: All outputs match reference model. |

| Functionality | FSM Coverage |
| --- | --- |
| - FSM coverage | Coverage reaches 100% Idle → Read transition Trigger: start_calc issued with valid start/end addresses. Expected: FSM moves from S_IDLE → S_READ. Read → Add transition Trigger: Operands fetched from SRAM_A and SRAM_B. Expected: FSM moves from S_READ → S_ADD. Add → Write transition Trigger: Sum computed. Expected: FSM moves from S_ADD → S_WRITE. Write → Read loop Condition: More operand pairs remain (rptr_q < read_end_addr). Expected: FSM loops S_WRITE → S_READ. Write → End transition Condition: No more operands remain (rptr_q >= read_end_addr and wptr_q >= write_end_addr). Expected: FSM moves from S_WRITE → S_END. End → Idle transition Trigger: Completion of all operations. Expected: FSM moves from S_END → S_IDLE. Reset from any state Trigger: Reset asserted during S_READ, S_ADD, or S_WRITE. Expected: FSM returns immediately to S_IDLE. Verified with directed reset tests. Conditional coverage – Write completes before Read Condition: (Test Case 8) Read pointer < end, but write pointer already at end. Expected: FSM resolves by finishing write path and cleanly terminating in S_END. Conditional coverage – Read completes before Write Condition: Write pointer < end, but read pointer already at end. Expected: FSM resolves by finishing read path and moving to S_END. |